A. Mobility aids and accessibility equipment

| ID | Scenario | Hazard / potential consequences | Proposed risk controls |
| --- | --- | --- | --- |
| LB-001 | Mobility aid battery placed in an emergency exit area | Battery fire, obstruction of evacuation, delayed access to exit | Prohibit unauthorized stowage in exit areas; confirm approved stowage location; cabin crew verification |
| LB-002 | Mobility aid battery removed and placed in overhead bin | Thermal runaway in inaccessible location; fire propagation to baggage | Verify battery configuration; protect terminals; use permitted stowage location; prevent crushing |
| LB-003 | Mobility aid battery remains attached to mobility aid during loading | Battery damage, short circuit, fire in cargo compartment | Verify handling instructions; secure mobility aid; apply manufacturer and operator procedures |
| LB-004 | Mobility aid battery exceeds applicable capacity limits | Non-compliant carriage; increased energy available during thermal runaway | Verify Wh or lithium content; confirm applicable mobility aid provisions; escalate unclear cases |
| LB-005 | Battery terminals exposed during handling | Short circuit due to contact with metal objects | Terminal protection; individual packaging where required; trained handling personnel |
| LB-006 | Battery damaged during removal from mobility aid | Internal damage, delayed thermal runaway | Trained personnel; approved removal instructions; stop handling if damage occurs |
| LB-007 | Battery is not removable or removal instructions are unavailable | Incorrect battery handling or configuration | Obtain manufacturer instructions; establish acceptance escalation; verify applicable carriage provisions |
| LB-008 | Multiple batteries fitted to one mobility aid | Incorrect battery count, configuration, or acceptance | Confirm battery count, type, capacity, and configuration before acceptance |
| LB-009 | Mobility aid battery becomes damaged during loading or unloading | Fire in cargo compartment or on ramp | Handling precautions; damage reporting; appropriate isolation and emergency response |
| LB-010 | Mobility aid battery fire occurs during deplaning | Ground personnel exposure, aircraft damage, evacuation disruption | Ground emergency response; crew coordination; appropriate response equipment and reporting |

B. Power banks and spare batteries

These scenarios should be among the highest priorities in the assessment because the battery may be carried independently of its original device and may be difficult to identify or access.

| ID | Scenario | Hazard / potential consequences | Proposed risk controls |
| --- | --- | --- | --- |
| LB-011 | Power bank placed in overhead bin | Delayed detection; inaccessible battery fire; smoke and fire spread | Establish approved stowage policy; passenger communication; crew access considerations |
| LB-012 | Power bank actively charges a phone during flight | Battery overheating; electrical fault; thermal runaway | Apply current power bank restrictions; prohibit onboard power bank recharging; passenger communication |
| LB-013 | Power bank is charging from aircraft power | Power bank overheating or failure | Apply current ICAO restrictions; prohibit charging power banks onboard |
| LB-014 | Power bank is damaged before boarding | Internal short circuit; thermal runaway | Passenger screening; damaged-device reporting; acceptance and escalation procedure |
| LB-015 | Power bank has no readable capacity label | Incorrect capacity determination; unauthorized carriage | Verify manufacturer documentation; establish acceptance escalation; reject if compliance cannot be confirmed |
| LB-016 | Passenger carries more power banks than permitted | Increased fire load; regulatory non-compliance | Passenger information; quantity verification; apply current passenger carriage limits |
| LB-017 | Power banks are stored together with metal objects | Short circuit; battery heating or fire | Individual protection; terminal protection; passenger information |
| LB-018 | Power bank is inadvertently activated inside a bag | Unintended electrical output; overheating | Protective storage; passenger awareness; prevent accidental activation |
| LB-019 | Power bank overheats during boarding | Passenger injury; smoke; need to stop boarding | Early reporting; trained personnel; appropriate emergency response |
| LB-020 | Spare lithium battery placed in checked baggage | Fire in cargo compartment; delayed detection | Gate-check controls; passenger communication; battery removal from baggage where required |
| LB-021 | Multiple spare batteries are carried in one bag | Increased fire load; multiple battery failures | Quantity limits; individual terminal protection; baggage acceptance controls |
| LB-022 | Battery is incorrectly classified as a power bank or spare battery | Incorrect carriage requirements applied | Classification guidance; Dangerous Goods escalation; staff training |

C. Portable electronic devices (PEDs)

| LB-023 | Passenger phone becomes trapped in seat mechanism | Battery crushing; internal short circuit; thermal runaway | Passenger instruction not to adjust seat; cabin crew intervention |
| --- | --- | --- | --- |
| LB-024 | Laptop battery overheats in overhead bin | Fire in inaccessible location; smoke; fire propagation | Accessible stowage policy; crew monitoring; passenger reporting |
| LB-025 | Laptop battery overheats while in use | Smoke or fire near passenger; crew exposure | Passenger reporting; emergency response; appropriate cooling and firefighting procedures |
| LB-026 | Tablet battery damaged by impact | Internal battery damage; delayed thermal runaway | Passenger awareness; damaged device reporting; safe handling |
| LB-027 | PED battery overheats while charging from aircraft power | Electrical fault; battery overheating | Apply approved charging restrictions; inspect damaged devices; passenger reporting |
| LB-028 | PED enters thermal runaway in a seat pocket | Smoke and fire in confined space; delayed access | Accessible device storage; crew response; emergency procedure |
| LB-029 | PED battery fire spreads to adjacent baggage | Fire propagation; increased severity | Fire response; removal of nearby combustible materials when safe; cooling |
| LB-030 | Battery fire occurs during taxi | Aircraft evacuation; passenger injury; operational disruption | Crew coordination; flight crew notification; emergency response and evacuation decision-making |
| LB-031 | Battery fire occurs during take-off or landing | Critical phase interruption; emergency landing or evacuation | Immediate crew response; flight crew coordination; emergency procedures |
| LB-032 | Battery fire occurs during flight | Smoke, fire, crew incapacitation, aircraft damage, diversion | Cabin crew emergency procedures; flight crew decision-making; appropriate firefighting and cooling |
| LB-033 | Battery fire occurs during deplaning | Passenger exposure; evacuation disruption; ground response | Cabin crew monitoring; ground emergency response; aircraft inspection |
| LB-034 | Device is damaged during gate-checking | Battery damage in cargo compartment | Remove batteries/devices as required; gate-check verification; passenger communication |

D. Other lithium battery-powered equipment

| ID | Scenario | Hazard / potential consequences | Proposed risk controls |
| --- | --- | --- | --- |
| LB-035 | E-cigarette or vape battery overheats | Fire, smoke, passenger injury | Carry-on-only requirements; prevent unintended activation; prohibit onboard charging and use |
| LB-036 | Smart luggage battery is non-removable | Incorrect carriage; battery fire | Verify battery configuration; apply smart baggage restrictions |
| LB-037 | Smart luggage battery is removable but not removed when required | Battery transported in checked baggage | Passenger communication; baggage acceptance checks |
| LB-038 | Rideable luggage or small electric vehicle is presented as a mobility aid | Incorrect classification; excessive battery capacity | Apply appropriate dangerous goods classification; distinguish mobility aids from personal transport devices |
| LB-039 | Drone battery carried in passenger baggage | Spare battery short circuit; incorrect capacity or quantity | Verify capacity; terminal protection; apply passenger carriage restrictions |
| LB-040 | Camera or professional equipment battery exceeds 100 Wh | Incorrect acceptance; unauthorized battery carriage | Verify Wh; operator approval where required; appropriate packaging |
| LB-041 | Crew portable equipment battery overheats | Crew injury; operational disruption | Crew equipment controls; approved carriage and use; emergency response |
| LB-042 | Medical device battery overheats | Fire; loss of medical function; passenger health impact | Medical equipment acceptance; contingency for loss of device; battery emergency procedures |
| LB-043 | Battery-powered device is accidentally activated in baggage | Overheating; electrical fault | Device protection; complete switch-off where applicable; baggage procedures |
| LB-044 | Lithium battery is incorrectly identified as non-dangerous goods | Non-compliant carriage; inadequate controls | Classification guidance; training; escalation to Dangerous Goods |

E. Emergency response and operational scenarios

These scenarios should be assessed independently from the battery acceptance scenarios. A battery may have been accepted correctly, but an event can still occur.

| LB-045 | Battery emits smoke without visible flames | Toxic fumes; delayed fire identification | Early reporting; crew emergency procedures; appropriate protective equipment |
| --- | --- | --- | --- |
| LB-046 | Battery enters thermal runaway inside overhead bin | Inaccessible fire; smoke; fire propagation | Detection; controlled access; appropriate firefighting and cooling procedures |
| LB-047 | Battery fire involves multiple batteries | Increased fire intensity; repeated thermal runaway | Appropriate response procedures; cooling; isolation where feasible |
| LB-048 | Battery fire reignites after initial extinguishment | Re-ignition; passenger and crew exposure | Continued cooling; monitoring; appropriate containment and inspection |
| LB-049 | Battery fire occurs near an emergency exit | Exit obstruction; evacuation hazard | Exit-area restrictions; cabin crew assessment; emergency evacuation procedures |
| LB-050 | Battery fire occurs near an oxygen source | Increased fire severity | Oxygen equipment controls; emergency procedures; appropriate separation |
| LB-051 | Battery fire occurs in a lavatory | Smoke spread; confined-space exposure | Smoke detection; cabin crew response; emergency procedures |
| LB-052 | Battery fire occurs in the flight deck | Flight crew distraction or incapacitation; aircraft control impact | Flight deck emergency procedures; equipment controls; crew coordination |
| LB-053 | Battery fire occurs during an emergency evacuation | Passenger injury; blocked evacuation route | Evacuation procedures; crew coordination; exit usability |
| LB-054 | Battery event occurs at a remote stand | Delayed emergency response; passenger exposure | Ground emergency response coordination; communication; evacuation considerations |
| LB-055 | Battery fire occurs during ground servicing | Ground personnel exposure; aircraft damage | Ground emergency response; equipment controls; reporting and investigation |
| LB-056 | Battery event is not reported after arrival | Continued exposure; failure to inspect aircraft | Mandatory reporting; post-event inspection; maintenance and safety notification |